Week 3 Project Progress Summary (Group Report)

1. Functionality Verified This Week

This week, our primary focus was on hardware integration and foundational performance testing. We successfully verified the following core functionalities:

Precise Line Following: The 8-channel infrared tracking module demonstrated stable and accurate recognition of both straight and curved paths, enabling reliable navigation.

Stable Motion Control: The MG513X DC geared motors, paired with the AT8236 dual motor driver, provided sufficient torque and speed control. The MG90S 180° servo ensured accurate steering adjustments.

Autonomous Navigation in Complex Paths: The integrated system (STM32F103C8T6 as the main controller) successfully completed tests on a composite track consisting of straight lines, 90° turns, T-junctions, and crossroads. The vehicle maintained trajectory without deviation or interruption, proving the basic synergy of hardware and control logic.

2. Current Main Uncertainties & Questions

While core functionality is established, we have identified several key uncertainties for the next phase:

Technical & Hardware:

Speed Optimization: Testing revealed an imbalance in speed performance—straight-line travel is relatively slow, while turning and crossing junctions are faster. We need to fine-tune the PID control algorithm and PWM parameters for more consistent speed management across all scenarios.

Advanced Perception Integration: The functionality of the K230 vision module for object recognition/detection has not yet been integrated into the main control loop. The communication protocol and data fusion strategy between the vision module and the STM32 need to be finalized and tested.

Sensor Fusion Reliability: We plan to use the HC-SR04+ ultrasonic module for obstacle avoidance. Its real-time responsiveness and potential interference with other sensors (e.g., infrared) in a dynamic environment require further validation.

Time & Task Division:

The integration of advanced AI features (using the K230 module) is on the critical path. We need to allocate dedicated time for algorithm adaptation and testing, which may impact the overall schedule if challenges arise.

Clear task ownership for debugging the speed control issue versus developing new features needs to be reassigned to maintain efficiency.

- Holiday Arrangements

- During the upcoming break, our group plans to:

Complete the hardware procurement of all remaining components as per the finalized budget.

Assign individual research tasks: One subgroup will delve deeper into PID tuning for motor control, while another will start working with the K230's development environment and example codes.

Schedule online sync meetings every week to review individual progress and adjust the plan for Week 4, ensuring we hit the ground running when the semester resumes.